ArCHER II N6004H

checklist (1/26)

PRE-FLIGHT CABIN

- Write-ups – Review / Verify

- Tach & Hobbs – Record

- VOR Test – Within 30 Days (IFR)

- Required Documents – On Board

- Parking Brake – As Req.

- All Switches – Off

- Autopilot Master – Off

- Magnetos – Off / Key Out

- Mixture – Idle Cutoff

- Tail Beacon – On

- Battery Master – On

- Avionics Master – On/Check Fan/Off

- Fuel Gauges – Check Quantity

- Pitot Heater – Check if Required (IFR)

- Interior Lights – Check if Required (night)

- Exterior Lights – Check

- Stall Warning – Check

- Ext Lights/Pitot Heat – Off (Beacon ON)

- Battery Master – Off

- Fuel Selector – Off then Least Full

- Flight Controls – Free and Correct

- Trim – Neutral

- Windows – Check Clean

- Flaps – Extend

- Tow Bar and Baggage – Stow & Secure

- Baggage Door – Close & Latch

PRE-FLIGHT RIGHT WING

- Surfaces – Check

- Flap and Hinges – Check

- Aileron and Hinges – Check

- Wingtip and Lights – Check

- Tie Down / Chocks – Remove & Stow

- Strut / Tire / Brake – Check

- Fuel Quantity – Check Visually

- Fuel Sump – Drain

- Fuel Cap – Secure

- Fresh Air Inlet – Check

PRE-FLIGHT NOSE

- Cowling – Secure

- Engine Oil – Check (Min 6 quarts)

- Dipstick & Filler Cap – Seated

- Propeller / Spinner – Check

- Cowling/Intakes – Check

- Alternator Belt – Check

- Fuel Sump – Drain

- Strut and Tire – Check

- Chocks – Remove & Stow

PRE-FLIGHT LEFT WING

- Surfaces – Check

- Fresh Air Inlet – Check

- Fuel Quantity – Check Visually

- Fuel Sump – Drain

- Fuel Cap – Secure

- Strut / Tire / Brake – Check

- Tie Down / Chocks – Remove & Stow

- Wingtip and Lights – Check

- Aileron and Hinges – Check

- Flap and Hinges – Check

PRE-FLIGHT FUSELAGE

- Antennas – Check

- Empennage – Check

- Stabilator & Trim Tab – Check

- Rudder – Check

- ELT – Check Plastic Screws

- Tie Down – Remove

PRE-FLIGHT FINAL INSPECTION

- 360° walk-around of Aircraft – Check

- Tie Down / Chocks – Remove & Stow

- Baggage Door – Close & Latch

PASSENGER BRIEFING

- Seatbelts, Shoulder Harness

- Door Operation

- Emergency Exit & Emergency Brace Positions

- Fire Extinguisher Use

- Eating, Drinking, Smoking, Conversation, Control Interference

- Collision Avoidance Assistance

- Physiological Considerations

- Supplemental Information

BEFORE STARTING ENGINE

- Preflight Inspection – Complete

- Passenger/Pilot Briefing – Complete

- Seats & Seatbelts – Adjust & Lock

- Flaps – Retract

- Pitch Trim – Neutral

- Rudder Trim – Neutral

- Alternate Static Source – Check then Off

- Circuit Breakers – In

- Carburetor Heat – Off (Cold)

- Fuel Selector – Least Full (L/R)

- Electrical Equipment – Off (Beacon ON)

- Radio Master – Off

- Autopilot – Off

- ELT – Armed

- Clock – Check Correct Time

STARTING ENGINE

- Brakes – Set

- Propeller – Visually Clear

- Rotating Beacon – On

- Master Switch – On

- Panel Lights – As Required

- Fuel Pump – On, Check Pressure

- Mixture – Full Rich

- Throttle – Cycle, then Open ½ inch

- Oil Temp – Check Cold or Hot

- Prime – Strokes (Cold 4-6 / Hot 1-2)

- Primer – Locked

- Propeller – Verbally “CLEAR!”

- Start Time – Recorded

- Magnetos – Start then Both

  - Throttle – 1000 RPM

  - Oil Pressure – Check

  - Alternator – On

  - Annunciator Panel – No Warnings

  - Fuel Selector – Change Tank

  - Ammeter – Charge Indicated

  - Mixture – Lean 1 inch

PRE-TAXI

- Fuel Pump – Off, Check Pressure

- Annunciator Panel – Test, All Lamps On

- Avionics Master – On

- Garmin GTN650

  - Database – Check

  - Fuel Quantity – Set (As Req.)

  - G5 HSI Test – Complete

- Autopilot Master – On (As Req.)

- Navigation Lights – On (As Req.)

- ATIS / ASOS / AWOS – Obtained

- Flight Instruments – Set and Check

  - G5 Heading – Check with Compass

  - G5 Altitude – Set, 75ft. Max

  - Altimeter – Set, 75ft. Max

  - VSI – Zero Noted

- Clearance – Obtain, Read Back

- Transponder – Code Set, Alt Mode

- Landing Light – As Req.

- Taxi Area – Clear

TAXI

- Brakes – Test (L/R/BOTH)

- Flight Controls – Wind Correction Set

- Instrument Check – IFR / VFR Check

  - Airspeed – Zero (or windspeed)

  - Compass – Check Fluid, Turning

  - G5 Heading – Turning

  - G5 – Proper Yaw, Opposite Slip

  - Turn Coordinator – Proper Yaw, Opposite Ball

ENGINE RUNUP

- Brakes – Set

- Mixture – Rich (As Req.)

- Flight Controls – Free & Correct

- Elevator Electric Trim – Test

- Elevator / Rudder Trim – Set for Takeoff

- Throttle – 2000 RPM

- Engine Gauges – Check

- Magnetos – Check – 175/50

- Ammeter – Check Charge

- Carb Heat – On and Check

- Throttle Idle – Check (575-675 RPM)

- Carb Heat – Off

- Throttle – 1000 RPM

- Throttle Friction – Set

- COM / NAV – Set

- Flaps – Set for Takeoff (0°-25°)

- Fuel Pump – On

- Parking Brake – Check Off

BEFORE TAKEOFF

- Windows and Doors – Closed & Latched

- Fuel Selector – Verify Fullest Tank

- Engine Instruments – Check

- Carb Heat – Verify Off

  - Transponder – Code Set, Alt Mode

  - Strobes / Landing Light – On

  - Departure Briefing – Complete

    - Runway – Confirm Direction

    - Crosswinds – Confirm Direction

    - Emergency Plan – Verbal Review

  - Takeoff Time – Recorded

NORMAL TAKEOFF

- Runway Heading – Verify with G5

- Rudder Pedals – Feet Off Brakes

- Throttle – Full Power

- Engine Gauges – Check

- Tachometer – Confirm Full Power

- Airspeed Indicator – Confirm Alive

- Airspeeds –

    - Vr 52-65 kts

    - Vx 64 kts

    - Vy 76 kts

CRUISE CLIMB

- Airspeed – 87 KIAS

- Flaps – Retract (As Req.)

- Fuel Pump – Off

- Landing Light – Off (If Desired)

CRUISE

  - Lights – As Req.

  - Throttle – 2000-2450 RPM (As Req.)

  - Mixture – Lean (75° Rich of EGT Peak)

  - G5 Heading – Check with Compass

  - Engine Instruments – Check

  - Fuel Selector – Manage Fuel Load

DESCENT / APPROACH

- ATIS / ASOS / AWOS – Obtained

- Altimeter – Set G5 and Analog

- Pre-Landing Brief – Complete

- Comm / Nav – Set

- Mixture – Enrich (As Req.)

- Fuel Selector – Fullest Tank

- Landing Light – On

BEFORE LANDING

- Autopilot – Disengage

- Trim – As Req.

- Carb Heat – As Req.

- Mixture – Full Rich (As Req.)

- Fuel Pump – On

- Seatbelts and Harnesses – On

- Flaps – As Required

- Carb Heat – On (As Req.)

- Landing Speed

  - Full Flaps – 60-70kts

  - No Flaps – 75kts

AFTER LANDING - (Clear of Runway)

- Flaps – Retract

- Carb Heat – Off (Cold)

- Pitot Heat – Off

- Landing/Taxi Light – As Req.

- Strobe Lights – Off

- Fuel Pump – Off

- Mixture – Lean 1”

N6004H – 01/26

Crosswind Max Demonstrated – 17

Vr – 52-65

Vx – 64

Vy – 76

Vso – 49

Vs – 54

Best Glide – 76

Cruise Climb – 87

Va – 1635 / 2550 lbs – 89 / 113

Vno – 125

Vne – 154

Vfe – 102

SHUTDOWN

- Transponder – VFR (1200)

- Throttle – 1000 RPM

- Elevator Trim – Neutral

- Autopilot Master – Off

- Avionics Master – Off

- Electrical Switches – Off (Beacon ON)

- Alternator Master – Off

- Mixture – Idle Cutoff

- MAGS – Off and Key Removed

- Battery Master – Off

- Magnetos – Off, Remove Key

- Tach / MX Squawks – Record

- Oil Change – Notify if Due Soon

- Chocks / Tie Downs – As Req.

- Flight Plan – CLOSED!

KCMI Frequencies

ATIS – 124.85	Clearance – 128.75

Ground – 121.8	Tower – 120.4

Approach/Departure – 121.35 (North/East)

Approach/Departure – 132.85 (South/West)

CMI VOR – 115.75